Friends of Leverett Pond

c/o Mitchell T. Mulholland

35 Cider Mill Road, PO Box 209

Leverett, MA  01054

Home 413-548-9161, cell: 413-531-2730

Email: mulholland@anthro.umass.edu

December 14, 2022

Jono Neiger, Chair

Adam Kohl, Administrator

Leverett Conservation Commission

PO Box 300

Leverett, MA  01054

Annual Report 2022

Leverett Pond Weed Management Project, Leverett, Massachusetts

Results of Herbicide Application, Notices and Surveys - 2022

MDEP Permit 200-0196

MDEP BRP License to Apply Chemicals WR04 No. WM04-0001007

Dear Jono, Adam and LCC members,

In 2022, following the guidelines in the Order of Conditions (200-0196) written by the Leverett Conservation Commission, the Friends of Leverett Pond (FLP) completed the weed management work as proposed under our application for Wetlands Protection Act permit 200-0196.

Results - Herbicide Application, Precautions, Pre/Post Treatment Surveys.

The following is a summary of weed management efforts conducted under Massachusetts Department of Environmental Protection (MDEP) permit #200-0196. This summary is submitted for Conservation Commission records.

- SWCA Third Party Mapping Survey. SWCA completed the “field work” for the pre-treatment, whole-pond mapping survey on June 30 and July 1, 2022. This is the “third-party” effort required by the Order of Conditions. SWCA analysts were on the pond for both days (Attachment 1). SWCA provided FLP with information on milfoil occurrences throughout the pond. One infestation was reported that also corresponded to a find reported by Brooke Thomas during FLP’s volunteer survey in April 2022. SWCA also mapped the extent of the large acre-size infestation at the north end of the public access channel. The resulting information was used to select and refine treatment areas and will serve for comparative purposes in annual surveys in future years. Photographs were taken and will be combined with the 2023 survey when that is done next June.

- FLP Volunteer Transect Survey. In April 2022, FLP held a training session for volunteers who were to undertake a transect survey to identify areas of milfoil on the Pond. FLP followed the suggestions made by Massachusetts Department of Conservation and Recreation for transect surveys. Following training, volunteers grouped into teams of two surveyors each, mounted kayaks and canoes, and mapped milfoil in pre-defined transects across the pond. The individuals who conducted the training were available on the pond to answer questions and to assist the survey teams. The information collected by the volunteer teams was considered along with the SWCA survey results and that of the FLP pre-treatment survey and contributed to treatment area selection for the proposed herbicide treatment planned for July 6, 2022.

- FLP Pre-Treatment/ Post-Treatment - Invasive Weed Annual Surveys. FLP conducted its annual pre-treatment survey on June 25, 2022, recording the percentage of invasive, nuisance and other weeds in pre-selected plots across the Pond (Attachment 2). The survey presently observes 37 plots (each approximately 4x4 meters in size) that are both in treatment areas, and untreated control areas. A 38th plot has been added to monitor an acre-size area in the south end of the pond that has a high infestation of milfoil. The results of the survey were used to inform the herbicide treatment plan, along with the SWCA survey and Volunteer Transect Survey. Prior to herbicide treatment, photographs were taken on July 3 and July 6 in the morning before treatment. Photographs were taken of existing invasives before they were treated with an herbicide. This was done to evaluate post-treatment conditions and the effectiveness of the herbicide treatment. A separate document concerning the 2022 annual survey is provided with the Annual Report submittal.

On October 6, 2022, FLP conducted a post-treatment weed survey of the pond following the proposed herbicide treatment. The FLP survey used is the system proposed by FLP in response to a 2017 request from the LCC and begun in 2018. The survey uses a GPS meter to assist in accurately locating 37 test and control points (Figure 1). Approximate weed percentages evident at each of the points were recorded on paper forms and are compared with weed conditions later in the season and in following years. Weed percentages were identified using a system developed by MDEP for terrestrial vegetation (Figure 2). The results indicate that milfoil dropped substantially from June 2022 by October 2022, three months  after the herbicide application. Compared with the 2019 ProcellaCOR treatment there was slightly less milfoil regrowth within four weeks.

- FLP Dissolved Oxygen Survey. FLP conducted a survey of dissolved oxygen at 16 locations in the pond. (Attachment 3).

DO evaluation in 2021 revealed that dissolved oxygen was showing depletion, to 0.00 mg/L in some cases and in milfoil-infested areas this year (2022) prior to treatment. In 2021 DO profiles exhibited sharp declines with increasing depth in most cases, and declines to at or near 0.00 mg/L at some locations. DO profiles in 2022 (approximately 3 months from the herbicide treatment) demonstrate a near elimination of this oxygen depletion phenomenon.

Dissolved oxygen concentration at or above 6 mg/L is known to support a healthy fish stock and is a key indicator of a healthy, oligotrophic pond. Results of testing from 2022 and 2021 are definitive indicators that: 1) Heavy milfoil infestation, driving high rates of biomass production and turnover, definitively leads to oxygen starvation in Leverett Pond; 2) selective removal of milfoil using a species-specific herbicide (ProcellaCOR) removes the infestation (see other portions of this report and Attachment 3) that is reflected in a definitive and dramatic improvement in the dissolved oxygen profile of the pond; 3) the increase in dissolved oxygen concentration in Leverett Pond throughout the water column following herbicide treatment is near-immediate (within a month following treatment); and 4) further research is required to quantify the sustained impact on dissolved oxygen concentration.

- 2022 Proposed Treatment Plan. In its proposal for weed management, FLP proposed two alternatives to treat the invasives on the Pond.

Proposed Treatment Alternative 1) Proposed was use of an “Eco Harvester ™.” This machine pulls milfoil by the roots using a roller system, conveys the pulled weeds into a hopper for off-loading and proper disposal on shore. The machine minimizes damage to the pond bottom. The method requires a “Dredging” permit from the MDEP Division of Waterways and has been applied for.

Proposed Treatment Alternative 2) If a Dredging Permit has not been issued yet (it can take up to 280 days) an herbicide application was proposed. This treatment is justified because of the increased growth in milfoil in shallow areas.  By July 2022 it was clear that the dredging permit would not be issued in time for effective milfoil management in the summer of 2022 (milfoil growing season) and the herbicide alternative was chosen. The alternative was also effective because of the considerable volume of milfoil that grew in three years of no weed management. Herbicide treatment reduces the volume of milfoil foliage thereby limiting the possibility of spreading by machine or wildlife.

- Licensed Applicator of Herbicide to Treat Milfoil. To conduct the herbicide treatment, FLP hired Water & Wetland LLC, a Lake/Pond Wetland Management Company (W&W), 115 South Street, Upton, MA 01568, Tel: 1-888-493-8526. This company is a registered applicator in the Commonwealth of Massachusetts (see licenses attached Figure 3). FLP had planned to hire Solitude Lake Management of Shrewsbury, but that company was unavailable. FLP was pleased with the professionalism and responsiveness of W&W personnel.

- License to Apply Chemicals. Prior to treatment, a License to Apply Chemicals to Control Nuisance Aquatic Vegetation WR04 No. WM04-0001007 was issued to W&W and FLP by the DEP Bureau of Resource Protection (Figure 4).

- Chemicals Used, EPA Registration Number, Amount Applied. Chemicals used by W&W to treat milfoil are as follows:

ProcellaCOR EC - EPA registration number 67690-80 (florpyrauxifen-benzyl) - 3.6 gallons applied;

Copper Sulfate - EPA registration number 46923-4, proposed if needed in License to Apply Chemicals, but not applied;

Captain XTR (liquid copper algaecide) – EPA registration number 67690,

3 gallons applied.

- Notification of Abutters and the Public. FLP submitted a notice of treatment to the Leverett Town Clerk and the Leverett Conservation Commission for posting on the town website (Figure 5). The document with a cautionary poster attached was posted on June 28, 2022, and is the top posted event on the website. FLP posted 40 high-visibility “dayglo-orange” signs (Figure 6) along Putney, Depot, Long Hill, Camp, and Cider Mill Roads; the Public Access/boat ramp (2); the Town Hall bulletin board; Leverett Library; Post Office bulletin board; the bulletin board at the Transfer Station (all June 20); the Long Hill Neighbors list-serve (many members use the pond), and a resident posted the message on the Leverett Connects list-serve. Two posters were posted at the Public Access, one on the informational kiosk and one on a wooden stake immediately to the left of the boat ramp at the waterline. Signs on wooden stakes also were posted on the immediate shorelines of waterfronts that are not regularly attended. Abutters and nearby residents were contacted in writing (postal), by email, and by phone. FLP feels that the notification process was successful. During the two-hour treatment, no one was seen on the pond. Only two shoreline individuals were observed, and they kept well away from the pond.

- Herbicide Treatment 2022. W&W treated the pond on July 6, 2022 with the ProcellaCOR herbicide (mixed to target milfoil and minimize any impact to other species). The company used a small aircraft-propellor-driven airboat to avoid submerged propellor damage that could result in spreading milfoil and other invasive plants. FLP members met the boat, assured that the vessel and trailer were weed-free and warned incoming boaters to avoid the pond for a day (there were only three during the treatment). Three orange traffic cones were located across the boat ramp - the central cone contained the precautionary poster. The W&W crew was provided with a treatment map by FLP with locations chosen based on the surveys (Figure 7). The actual treatment was controlled using a GPS system and shown on an aerial photograph of the Pond (Figure 8). The FLP members were joined by the Leverett Police Department to handle any traffic problems that might arise. Prior to the herbicide treatment, an FLP member accompanied the boat operator around the pond to physically identify the treatment areas. One offshore area was identified late in the process and was shown to the operator so that it could be treated. Once the treatment started, an FLP member followed the boat at a distance to answer any questions should they arise, assure that the correct areas were treated, and to warn off any curious boaters or swimmers (there were none).

The herbicide applicator W&W, LLC, conducted one herbicide application on July 6, 2022 using the herbicide ProcellaCOR and an algaecide (Captain XTR Copper - see below) to control Variable and Eurasian Watermilfoil. The algaecide discouraged algal blooms caused by weed decomposition and hot weather. Substantial algae was already in growth throughout the pond in high infestation areas. Note that dry Copper Sulfate was included in the License to Apply Chemicals if needed. Dry Copper Sulfate was not applied. Methods for management of invasive weeds and nuisance vegetation in Leverett Pond are outlined in the Order of Conditions for MDEP Wetlands Protection Permit 200-0196, and were followed by the FLP. The ProcellaCOR herbicide was applied via water column injection using underwater hoses by the airboat. The herbicide was not applied as a surface spray. Using this technique the boat and aircraft propellor are well away from the area treated before the herbicide can surface, thereby eliminating spray to other areas, including the shoreline.

The treated area on Leverett Pond was approximately eight acres of water that included town Public Access/Right–of-Way, two areas of infestation offshore, and shallow-water shoreline waterfront areas. Areas that were treated extended from as close to the shore as possible in indicated areas (Figure 7) generally within 30-40 feet from the shoreline and outward approximately 75 feet. The treated areas were as proposed in the FLP 2022 proposal (NOI) and updated with information from the SWCA mapping survey and FLP survey. The attached maps (Figures 7 and 8) shows the areas of treatment. Also, for LCC convenience attached is the MDEP Bureau of Resource Protection License to Apply Chemicals DEP BRP WR04 No. WM04-0001007  (Figure 4). Note that one area in the southwestern portion of the pond was observed late in the process, was treated by W&W and was added to the GPS W&W aerial photo/map by FLP (Figure 8). This area had been identified during an FLP survey the morning of the application.

- Effectiveness of ProcellaCOR. ProcellaCOR has been designed specifically to control milfoil, and uptake is so efficient that residual herbicide is not detected within 24-72 hours. The herbicide is mixed at a very low dosage designed to specifically kill milfoil and to have minimal impact on other species.  Because of its effectiveness, re-use the following year is not generally recommended and in some cases relief from regrowth can last up to three years. The herbicide has been used throughout the United States and notably in Minnesota (state motto: “Land of 10,000 Lakes”) with great success. Permitted in Massachusetts for use in agriculture, the herbicide was only permitted for use in waterbodies in 2019.

- Restrictions to Public Pond Use. Restrictions following herbicide application for most activities were for one day. No swimming (people or pets [FLP recommended the latter]), boating, or fishing until July 7. No irrigation was recommended until July 13. There were no restrictions for “drinking & cooking, livestock/domestic animal consumption.” The posters remained in place until July 14 - a day after the irrigation restriction is lifted (Figure 6).

- Post-Treatment Inspection by Applicator. As required in the Order of Conditions, within four weeks of the treatment date of July 6, the treatment areas were inspected by W&W personnel. A GPS controlled map of treatment areas was provided by the applicator (Figure 8). Only one tiny area of remaining milfoil was observed (see report of W&W follow-up with accompanying maps). The herbicide treatment was determined to be effective. FLP also conducted a photographic survey of treated areas on August 3, 2022.

- Post-Treatment Analysis of Water Samples. Water samples for residual ProcellaCOR were collected by FLP at the Public Access, and the northwest cove. The samples were shipped by overnight courier to the SePro laboratory in Whitakers, NC for analysis. Results show that remnant ProcellaCOR was below detectable (Attachment 4).

- Results of 2022 Herbicide Treatment. The ProcellaCOR herbicide application was extremely effective in controlling milfoil. Within 10 days, deteriorated below-surface Variable and Eurasian watermilfoil (Myriophyllum heterophyllum and Myriophyllum spicatum) were observed, and the milfoil foliage was dead and fragmented. The algaecide had very effectively eliminated the algae that was so prevalent before treatment. This is especially the case at the Public Access which had an alarming growth of algae mixed with invasive plants. The inspection by the W&W operator four weeks after the treatment found only one remnant plant on the east shore of the pond (west of the Neyman property) (Attachment 5). A survey by FLP also found a small growth off the Congregational Church waterfront (E02-T). By the end of the 2022 season on October 6, four small areas of milfoil growth were observed. They include the two areas mentioned above, and a few intermittent growths in treated areas W-05 and W-06 approximately 150 feet east of the west shore. This is an area of very high milfoil infestation that has been growing steadily for the past 2 years. Also, a small clump of milfoil was observed near the shore of W-04T on the west side. This is within an area of very dense pondweed, bladderwort and floating leaf vegetation. Speculatively, the strength and flow of the herbicide may have been reduced through heavy growth non-targeted weeds in this area.

In 2019 it was suggested that a ProcellaCOR treatment resulted in the creation of “floating islands,” even though most of the “islands” were far distant from the treated areas. This issue was discussed by FLP at that time, maintaining that evidence of floating islands as far back as the 1930s exists, and that low water levels resulting from the leaking dam contributed not to the islands themselves, but to their increased visibility. Interestingly, in 2022 there were far fewer islands following the ProcellaCOR treatment than in earlier years (with or without treatment).  Photographs support this. FLP believes this is the result of normal water levels on the Pond resulting from replacement of the dam. FLP has provided the LCC with photographs of “before and after” treatment. In that folder there are photographs labelled “FYI Islands” showing islands that occurred before treatment.

Very limited impacts to non-targeted species were observed following the ProcellaCOR treatment. In 2019, watershield (a nuisance floating-leaf weed) was temporarily effected within about 15 feet of the treated areas. This was not seen in 2022.

Results of Hand Raking and Pulling. FLP proposed hand-pulling and raking occurred as a follow-up to eliminate or retard re-growth of nuisance weeds. SCUBA was not attempted this year, but remains a possibility in the future especially following the efficiency of the ProcellaCOR application where regrowth is of low volume.  Hand-pulling was conducted by several landowners to avoid milfoil re-growth. This is an effective weed management method, but the area covered is very small.

- Results of Benthic Barrier use.  As an alternative to herbicide treatment and mechanical raking, four landowners continue to experiment with benthic barriers (approximately 12 by 24 feet in size). The cloth is placed on the bottom where herbicide treatment and hydro-raking had occurred. Milfoil does not grow through the barrier. This is an effective method of weed control, but the barriers require maintenance and only a small area can be controlled. Gas bubbles occasionally must be released. This problem is minimized if the barriers are placed immediately after hydro-raking when installed. The barriers require cleaning or re-setting to avoid the accumulation of silt which will support plant growth.

- Volunteer Weed Management at the Public Access. In order to remove detritus and growth of nuisance plants at the Public Access a group of FLP volunteers wielded rakes and other tools to clear dead plants and invasive floaters. Removed materials were loaded onto a trailer and deposited in the Leverett Transfer Station.

In summary, by October 6, 2022, milfoil was effectively reduced in volume and area and algae was controlled in areas treated. Impact on non-targeted species was not observed. In 2019, some temporary reduction of leaves and stalks of watershield (a nuisance weed) was observed up to 15 feet from the treatment area. In 2022, this was not observed.

Surveys show very little curly-leaf pondweed on the pond in 2022. There are occasional very small growths of curly-leaf regrowing at the Public Access, but they were very low volume in October and could be removed easily by raking. Interestingly, in 2022, Robbins Pondweed was observed in several areas of the Pond especially off the north and northeast shorelines. This plant was very common in the 1970s and 1980s, but has been seen only rarely in recent years. It is possible that the reduction of milfoil thickets allowed the plant to rebound. Generally, Robbins Pondweed is beneficial because under most conditions it is not monoculturistic, provides cover and access to the bottom to fish, and rarely becomes emergent.

There is continued colonization in both near-shore and deeper portions of the pond of Large Leaf Pondweed (Potomogeton Amplifolius). The plant occurred suddenly following two years of prolonged low water that resulted from a leaking dam. Large Leaf Pondweed is a native plant and can be cyclical like Bladderwort, but is a nuisance and is prone to monocultural growth in places. Mechanical control is recommended.

Other Issues

Education to Avoid Spread of Weeds. FLP continues to maintain an educational and informative kiosk at the Public Access. This kiosk was authorized by the Select Board and Conservation Commission in the 1980s. Posters inform the public about avoiding bringing milfoil in and out of Leverett Pond on propellers and boat trailers. FLP is seeking a brochure produced by the Mass DCR to put on vehicle windshields at the Public Access. In 2020-2022 a COVID safety poster was added to the board and remains today. FLP also maintains an informative web site friendsofleverettpond.org with information concerning weed management and other activities of the group.

Boat Loan Program. FLP has continued the free boat-loan program to Leverett Residents that encourages the public to enjoy and appreciate Leverett Pond. Leverett Public Library staff made available a table every Saturday morning for FLP volunteers to sign out keys for kayaks and canoes to residents for week-long use. Boats and gear are kept at a boat rack and storage box at the Public Access. It was an amazing summer for pond paddlers with nearly 200 key checkouts. The Friends of Leverett Pond is happy to make this available and thanks all our volunteers without whom none of this would be possible.  Special thanks to the library staff - Hannah, Sue, and Jill - who were most welcoming and supported our program beyond what was necessary.

Pond Conditions from Dam Replacement. In 2022, the water level on the pond was usually near normal. There were some problems during the summer when floating mats of bladderwort repeatedly clogged the outlet raising water levels about 8 inches above normal. This was remedied by a dedicated FLP volunteer who unclogged the outlet and kept the water flowing. FLP is discussing remedies for the problem -- Possibly a rope with floats across the channel upstream of the dam that would catch floating weeds. Volunteers periodically would remove the weeds stopped by the rope.

Potential Future Projects. There are other projects being considered. They include the possible construction of a wetland walkways into the pond from near the Public Access, and possibly in the north end of the Pond.  There have been discussions of making a suitable porta-potty available somewhere appropriate. All projects will be presented to the Conservation Commission, and necessary permits applied for before construction would take place.

Tentative Plans for 2023 Weed Management

The FLP is presently working on a weed management plan/proposal for the upcoming 2023 season. The plan will be submitted to the LCC by February 15, 2023 as required by the Order of Conditions. Currently the FLP is considering use of an Eco-Harvester™ or mechanical harvester to continue invasive weed management, provided that a dredging permit is received.

If you require additional information, you can reach me at home (413-548-9161) or cell (413-531-2730). My E-Mail address is mulholland@anthro.umass.edu. I would be happy to meet with you whenever you wish.

Sincerely,

Mitchell T. Mulholland

VP Lake Weed Management

Friends of Leverett Pond

List of Figures and Attachments:

Figure 1.  FLP Areas to be Surveyed – with treatment and control plot codes

Figure 2.  DEP Key for Determining Vegetation percentages

Figure 3 Applicator’s Licenses

Figure 4 License to Apply Chemicals

Figure 5 Public Notice to Town for posting

Figure 6 Precautionary Poster – 40 posted

Figure 7 FLP Treatment Map Provided to Applicator

Figure 8 Treatment Map Provided by W&W Applicator

Attachment 1. SWCA Mapping Survey Report with Maps

Attachment 2 FLP survey results and graphs for 2022

Attachment 3 FLP DO results

Attachment 4 Pond Chemistry Following Treatment

Attachment 5 W&W Field Notes and Post Treatment survey results