| LEVERETT POND AQUATIC PLANT ASSESSMENT REPORT 2022 September 2022 |  |
| --- | --- |
| PREPARED FOR Friends of Leverett Pond |  |
| PREPARED BY SWCA Environmental Consultants Project 64784 |  |

Leverett Pond AQUATIC plant assessment Report

2022

Prepared for

The Friends of Leverett Pond

c/o 35 Cider Mill Road

PO Box 209

Leverett, Massachusetts 01054

Attn: Mitch Mulholland and Tom Hankinson

Prepared by

Naomi Valentine, M.S., CERP, Christin McDonough, M.S., CWB, PWS, CESSWI, Mickey Marcus

SWCA Environmental Consultants

15 Research Drive

Amherst, Massachusetts 01002

(413) 256-0202

www.swca.com

SWCA Project No. 00064784-000-AMH

September 2022

This page intentionally left blank.

Contents

1	Introduction	1

2	Site Description	1

3	Aquatic Plant Assessment	1

3.1	2022 Survey Results	2

4	Summary	4

Appendices

Appendix A. Figures

Appendix B. 2022 Aquatic Vegetation Survey Data

Tables

Table 1. Leverett Pond Aquatic Vegetation Survey – Target Vegetation Distribution	2

Table 1a. Aquatic Vegetation Prevalence Analysis – Quantity of Quadrats by Percent Cover	3

Table 2. Leverett Pond Aquatic Vegetation Survey – Biomass Index	3

This page intentionally left blank.

# Introduction

The Friends received an Order of Conditions (OOC) from the Leverett Conservation Commission (DEP File No. 200-0196) in 2022 for the management of nuisance aquatic vegetation. As part of that OOC, the Friends need to conduct an annual pre-management survey of aquatic vegetation in the pond. SWCA completed this survey on June 30 and July 1, 2022, prior to any management of submerged aquatic plants within the pond. The purpose of this report and associated recommendations is to describe pre-management condition of aquatic vegetation in Leverett Pond for the 2022 season.

# Site Description

The pond is approximately 102 acres in surface area and was classified as a “Great Pond” in 1994. The reported maximum depth within the pond is 21 feet and the average depth was recorded as 5 feet by the Massachusetts Division of Fisheries & Wildlife (2016). Most of the pond is a warm water lacustrine system, a littoral subsystem, and an aquatic bed class (Cowardin et al., 1979). The margins of the pond include palustrine forested wetland, palustrine shrub wetland, and palustrine emergent wetland classes (both persistent and non-persistent communities were observed). The pond substrate consists of muck and sand; however, aquatic vegetation covers more than 40% of the bottom of the pond and submergent vegetation covers nearly 100% of the bottom. Aside from the vegetative buffers to the pond, it is surrounded by low-density residential land use and forest.

A public right-of-way (ROW) paved boat ramp with limited parking is located at the southern end of the pond at 190 Depot Road in Leverett. Small streams drain into the southern end of the pond by Depot Road and the margins of the pond near the boat ramp are densely vegetated with broad-leaf cattails (Typha latifolia) and other emergent plants. The pond flows to the north into a diverse swamp and outlets through a dam on the northeast corner of the pond.

Two historic mills were located downstream of the northern end of the pond in 1794 (Acorn Botanical Associates, 1999). A concrete dam located at the northeastern end of the pond was originally constructed in 1938 and repaired in 1952, 1979, and completely replaced 2020. The pond water level has been manipulated by past drawdowns, such as the drawdowns performed in the 1950’s (Acorn Botanical Associates 1999). Additionally, the pond level was 24 inches below its normal level in 2018 and 2019 due to dam leaks, which have since been repaired.

# Aquatic Plant Assessment

SWCA conducted an aquatic plant survey within the pond on June 30, 2022, and July 1, 2022. The survey was conducted from a Jon boat, using a point-intercept survey in which each point was associated with a 200 by 200-foot quadrat. The quadrats and their center points were located using Field Maps for ArcGIS on a tablet with a Geode antenna receiver. The survey within each quadrat was conducted via visual and rake-toss observations. The surveyors identified the total coverage of all vegetation observed, target (invasive or nuisance) vegetation, as well as each individual species observed. These data are represented in Table 1 and in Figures 1 – 5, located in Appendix A of this report.

The dominant species identified within the pond included both invasive and native species, with the majority of aquatic species consisting of water lily and invasive watermilfoil. Species include: Eurasian watermilfoil (Myriophyllum spicatum), robbins pondweed (Potamogeton robinsii), curly-leaf pondweed (Potamogeton crispus), large-leaf pondweed (Potamogeton amplifolius), watershield (Brasenia schreberi), common waterweed (Elodea canadensis), yellow pondlily (Nuphar variegata), white waterlily (Nymphaea odorata), longleaf pondweed (Potamogeton nodosus), purple bladderwort (Utricularia purpurea), swollen bladderwort (U. inflata), and common bladderwort (Utricularia vulgaris). Other plant species that have been identified in the pond in the past include fanwort (Cabomba caroliniana), variable-leaf watermilfoil (Myriophyllum heterophyllum), free-flowered waterweed (Elodea nuttalli), Common Cattail (Typha latifolia), brittle naiad (Najas minor), and ribbon-leaf pondweed (Potamogeton epihydrus).

## 2022 Survey Results

Tables 1 and 2 present the results of the 2022 aquatic vegetation survey within Leverett Pond in two different ways. Table 1 shows the presence of various target vegetation within set ranges of percent cover across all quadrats surveyed. This provides a snapshot of relative densities of the target species within Leverett Pond. Table 2 presents relative biomass density throughout the water column in Leverett Pond on the days of the survey for each quadrate surveyed. Additionally, Table 2 presents what the average percent cover of both Eurasian watermilfoil and of all vegetation within those same quadrats. These data are presented to add a third dimension to the data collection; presenting an understanding not only two-dimensional percent cover, but also density down through the water column.

Table 1. Leverett Pond Aquatic Vegetation Survey – Target Vegetation Distribution

| Target Species | Target Sp. % Cover | Quadrat Number(s) |
| --- | --- | --- |
| Eurasian water milfoil | 85% - 90% | 37,65,78 |
| Eurasian water milfoil | 75% - 80% | 85, 93, 97 |
| Eurasian water milfoil | 45% - 65% | 17, 45, 56, 64, 95, 105* |
| Eurasian water milfoil | 30% - 40% | 16, 21, 32, 41, 50, 80, 83, 91 |
| Eurasian water milfoil | 20% - 25% | 24, 25, 26, 101, 102, 103, 105* |
| Eurasian water milfoil | 10% - 15% | 33, 46, 61, 67, 74, 76, 84, 94 |
| Eurasian water milfoil | 5% | 20, 31, 36, 38, 42, 48, 55, 59, 60, 63, 69, 70, 77 |
| Curly-leaf pondweed | 5% | 84, 105* |
| Robbins pondweed | 70% - 80% | 85, 86 |
| Robbins pondweed | 45% - 65% | 25, 28, 30 |
| Robbins pondweed | 10% - 15% | 21, 31, 35 |
| Swollen bladderwort | 70% - 80% | 19, 20, |
| Swollen bladderwort | 45% - 65% | 27, 53, 74 |
| Swollen bladderwort | 30% - 40% | 43, 47, 62, 66, 70, 77, 89 |
| Swollen bladderwort | 20% - 25% | 17, 31, 54, 73, 95, 105* |
| Swollen bladderwort | 10% - 15% | 18, 21, 28, 30, 33,42, 46, 48, 61, 67, 71*, 72, 79*, 80, 81, 82, 83, 84, 88, 91*, 92 |
| Swollen bladderwort | 5% | 22, 32, 50, 59, 68, 75, 86*, 87, 93 |
| Water lily** | 85% - 90% | 1,3,4,5,6,7,8,9, 10, 11, 12, 13, 14, 15, 51, 57, 58, 71, 79, 90, 96, 98, 99, 100, 104, 106, 107, 108 |
| Water lily** | 70% - 80% | 86, 101, 103 |
| Water lily** | 45% - 65% | 16, 22, 38, 52, 87, 92, 94, 102 |
| Water lily** | 30% - 40% | 64, 72, 105 |
| Water lily** | 20% - 25% | 17, 32, 36, 69, 84 |
| Water lily** | 10% - 15% | 18, 21, 25, 27, 66, 77, 80, 82, 85, 93, 97 |
| Water lily** | 5% | 20, 65, 76, 78, 83, 91, 95 |

Note: Polygons are represented in Figure 1. These data include aquatic vegetation only; no emergent or other vegetation types are included.

* These Sampling Unit includes some observations that were not noted during the original survey and are not indicated on the figures, but the plants noted should be included as noted in this table.

** Water lilies were identified as target vegetation in past years. These areas identified include a mix of watershield and water lilies as well as largeleaf pondweed in locations.

Aquatic plants were noted at some density within 101 of the 108 quadrats surveyed in Leverett Pond. As seen in the attached figures and Table 1, waterlily species are the most prevalent (widespread) across Leverett Pond. Lilies were noted in 66 of the 108 quadrats within the pond. Eurasian watermilfoil is the most prevalent submerged aquatic plant within Leverett Pond. It was observed in 47 of the 108 quadrats. Twenty of those observations were noted at 25% cover or more and 8 were noted at 50% cover or more. The next most prevalent submerged aquatic plant species is swollen bladderwort, which was noted in 43 of the 108 quadrats and noted at greater than 25% and 50% cover within 12 and 4 quadrats, respectively. Table 1a shows these prevalence notes for all species surveyed as well as notes on total vegetative cover.

Table 1a. Aquatic Vegetation Prevalence Analysis – Quantity of Quadrats by Percent Cover

|  | Any Plants | Any Target Species | Robbins Pondweed | Eurasian Watermilfoil | Swollen Bladderwort | Waterlily |
| --- | --- | --- | --- | --- | --- | --- |
| Any Coverage | 101 | 101 | 8 | 47 | 43 | 66 |
| > 25% | 100 | 80 | 5 | 20 | 12 | 43 |
| > 50% | 91 | 62 | 4 | 8 | 4 | 33 |
| > 75% | 53 | 45 | 0 | 5 | 0 | 28 |
| > 89% | 41 | 41 | 0 | 1 | 0 | 28 |

Note: Full data set presented in Appendix B. These data include aquatic vegetation only; no emergent or other vegetation types are included.

As seen in Table 2, the vast majority of the pond is not only vegetated, but densely vegetated throughout the water column. More than half of the pond contains submerged aquatic vegetation that fills the water column (where located) and overtop or cover the surface of the pond. There are only 7 quadrats in which no vegetation was observed and only 1 quadrat in which biomass was low. Therefore, not only does Leverett Pond contain a widespread cover of aquatic vegetation, but the vegetation is also dense throughout the water column and/or covers the surface of the pond where it is present.

Table 2. Leverett Pond Aquatic Vegetation Survey – Biomass Index

| Biomass | Quadrat Number(s) | Quantity of Quadrats | Average % Cover of Milfoil | Average % Cover of all vegetation |
| --- | --- | --- | --- | --- |
| Very low growth | 35 | 1 | 0% | 10% |
| Growth Extending up Into Water Column | 23, 24, 28, 30, 42, 45, 47, 48, 53, 54, 56, 61, 62, 67, 73, 74, 95 | 17 | 15% | 64% |
| Growth in Water Column and Possibly to Surface | 18, 19, 20, 21, 25, 26, 27, 31, 32, 33, 36, 37, 43, 46, 55, 59, 63, 66, 68, 69, 70, 75, 76, 77, 80, 88, 89 | 27 | 12% | 67% |
| Growth Filling the Water Column and Covering the Surface | 1, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 22, 38, 41, 50, 51, 52, 57, 58, 60, 64, 65, 71, 72, 78, 79, 81, 82, 83, 84, 85, 86, 87, 90, 91, 92, 93, 95, 96, 97, 98, 99, 100, 101, 102, 103, 104, 105, 106, 107, 108 | 56 | 15% | 83% |

Note: Polygons are represented in Figure 1. These data include aquatic vegetation only; no emergent or other vegetation types are included.

# Summary

This pre-management survey for the 2022 season is meant to record the density and cover of aquatic vegetation prior to any management activities. These data will be compared to post-management coverage in a follow-up survey at a similar time of year in 2023. The results of this survey indicate that aquatic vegetation (both submerged and floating) cover the vast majority of Leverett Pond in late June to early July. Previous years’ data indicate that this is the case for most of the year, but that the relative densities of vegetation present shift (i.e. most pondweeds are more dense than Eurasian watermilfoil earlier in the growing season).

This page intentionally left blank.

Appendix A

Figures

Appendix B

2022 Aquatic Vegetation Survey Data