Dissolved Oxygen Profiles

Dissolved oxygen concentration (DO) measurements were recorded at various locations and at various depths throughout the pond both before, and following, milfoil treatment with ProcellaCOR. During the first week in July, 2022 the pond was screened to determine if the profiles typical of 2021 persisted into 2022.

Prior to ProcellaCOR Treatment. Below is a table of eight profiles, seven collected along a parallel leading from off the Hankinson shore (east side center) to off the Hopley shore (west side center). That parallel includes both shorelines as well as shallow, heavily infested areas, and deep, non-infested areas. The eighth sample was collected off the Friedman/Ryan shore (northeast cove) where milfoil and other aquatic plants are especially dense. DO was measured at two depths – one foot and six feet.

Sample Location	Water Depth (feet)

1	6

________________________________________________

1	7.44	5.38 (actually 4 feet – max. depth)

2	7.23	0.12

3	7.57	0.46

4	7.60	0.22

5	8.35	8.40

6	8.18	8.90

7	8.21	7.90

8	7.97	0.00 (actually 4 feet – max. depth)

Post-ProcellaCOR Treatment. Dissolved oxygen profiles were collected September 11, 2022, 60 days following ProcellaCOR treatment (see Oxygen/Temperature Profiles attachment). Eighteen profiles were collected, each profile including as few a one measurement, due to shallow water, to seven measurements at one-foot-depth intervals. DO and temperature readings were collected at each location and depth.

No milfoil was observed at any of the sampling locations. Trace milfoil was observed at discrete locations along the Hankinson, Hosley and Church Lot shorelines (Hankinson personal observation)

With one exception, dissolved oxygen concentration did not vary appreciably with water depth.

With three exceptions, concentrations ranged from 6 to 10 mg/L at the one-foot depth and remained near the one-foot-depth concentration down through the water column.

The four exceptions:

- Profile “D. At Public Access Channel Opening (2)”: DO declined with depth, beginning 6.45 mg/L at one-foot depth to 5.28 and 1.62 mg/L at two- and three-foot depths, respectively. This decline was not supported by Profiles C and E that were collected nearby. These samplings were taken in an area of dense milfoil prior to treatment.

- Profiles “M”, “O”, and “R”: These profiles (not really profiles as the water depth allowed for only a single measurement) were collected at different locations in the middle of the channel leading to the boat launch area. [DO] was detected at a single, one-foot depth, at the sediment surface. DO ranged from 2.78 to 3.95 mg/L.

Interpretation

Dissolved oxygen was showing depletion, to 0.00 mg/L in some cases, last year (2021) and in milfoil-infested areas this year (2022) prior to treatment. Last year (2021) DO profiles exhibited sharp declines with increasing depth in most cases, and declines to at or near 0.00 mg/L at some locations. DO profiles this year demonstrate a near elimination of this oxygen depletion phenomenon.

Dissolved oxygen concentration at or above 6 mg/L is known to support a healthy fish stock and is a key indicator of a healthy, oligotrophic pond. Results of testing from this year (2022) and last year (2021) are definitive indicators that:

1]	Heavy milfoil infestation, driving high rates of biomass production and turnover, definitively leads to oxygen starvation in Leverett Pond;

2]	Selective removal of milfoil using a species-specific herbicide removes the infestation (see elsewhere in the 2022 Annual Report) that is reflected in a definitive and dramatic improvement in the dissolved oxygen profile of the pond.

3]	The increase in dissolved oxygen concentration in Leverett Pond throughout the water column following treatment is near-immediate [within a month following treatment].

4]	Further research is required to quantify the sustained impact on dissolved oxygen concentration.